Computational seismology

Acoustic wave propagation

Either Matlab:

Download the Matlab Program ac2d.m .

Open Matlab in a Command Window .

Try to run the model by typing

>> ac2d

in the working directory

Or Python:

Download the Python Program ac2d.py .

Open ipython -pylab in a Windows cmd window .

Try to run the model by typing

In[1] run -i ac2d.py

in the working directory

Tasks:

- Determine numerically the stability limit of the code as accurately as possible by varying eps.

- Increase the frequency of the wavefield by varying f0. Investigate the angular dependence of the wavefield. Why does the wavefield look anisotropic. Which direction is the most accurate and why?

- Modify the program such that at the end of the calculation you can visualize and output synthetic seismograms.

- Modify the velocity model c and observe the resulting wavefield.

- Add a low(high) velocity layer near the surface. Put the source at zs=2.

- Add a vertical low velocity zone (fault zone) of a certain width (e.g. 10 grid points), and discuss the resulting wavefield

- Simulate topography by setting the pressure to 0 above the surface. Use a Gaussian hill shape or a random topography.

- Use a spike source time function and look at the resulting seismogram

- Examine the spectrum of    the “Green’s function”. Do you spot the numerical noise?

- Concolve the Green’s function with an appropriate source time function (e.g., a Gaussian of appropriate length). What happens with the numerical noise?